Adrenal Gland Segmentation — Experimental Results

Stage B segmentation, Stage A slice gating, and the spiking-network evaluation

M. Fayemiwo  ·  31 August 2026  ·  AMOS22, CT (200 training / 100 validation patients)

# 1.  Objective and setup

The pipeline is a two-stage cascade. Stage A decides which axial slices contain adrenal tissue; Stage B segments only those slices. The research question is whether presenting k consecutive slices to a spiking neural network as k discrete time steps — so that membrane potential integrates evidence along the craniocaudal axis — outperforms a matched-capacity recurrent baseline given identical inputs.

All experiments use AMOS22 CT: 200 training and 100 validation patients, volumes resampled to 1.0 mm in-plane and 2.5 mm through-plane so that a fixed k-slice window spans the same physical distance for every patient, HU-windowed and centre-cropped to 384 px, with left and right glands as separate output channels. Segmentation is reported as mean per-case Dice — each patient scored over the whole volume, then averaged. The reference point throughout is Fayemiwo et al., Journal of Imaging Informatics in Medicine 2025, which used the same dataset and the same metric definition, making the comparison direct.

# 2.  Stage B: segmentation performance

The segmenter reaches mean per-case Dice of 0.748 (left) and 0.747 (right) over 100 validation patients. This is below the 2025 pipeline on both glands.

|  | L.A.G raw | L.A.G post | R.A.G raw | R.A.G post |
| --- | --- | --- | --- | --- |
| This work | 0.746 | 0.748 | 0.745 | 0.747 |
| 2025 paper, VGG16 | 0.820 | 0.910 | 0.750 | 0.900 |
| 2025 paper, nnU-Net (2D) | 0.850 | 0.920 | 0.820 | 0.910 |
| 2025 paper, TransUNet | 0.880 | 0.920 | 0.830 | 0.930 |

Table 1. Mean per-case Dice. Against the strongest prior model the deficit is 0.134 on the left gland and 0.085 on the right.

The distribution matters as much as the mean. Median Dice is 0.784 against a mean of 0.755, so a small number of patients carries the deficit: 6 of 200 gland instances score below 0.5. Repeating an evaluation of the same checkpoint at the same operating threshold gives means differing by about 0.008, which sets the resolution of every comparison in this document — differences smaller than that are not treated as real.

# 3.  What determines Stage B performance

Four candidate explanations for the failing cases were tested directly. Each was measured rather than assumed, and none survived.

| Factor | How it was tested | Result |
| --- | --- | --- |
| Field of view | Native gland bounding box mapped through the same resample and 384 px crop | Not a factor — all glands inside, 95–102% of voxels retained |
| Left/right confusion | Each prediction scored against the opposite gland's ground truth | Not a factor — 0.000 in every case |
| Threshold calibration | Per-case oracle threshold against one global threshold | Marginal — worth +0.010 Dice at best |
| Slice thickness | Dice stratified by native z-spacing | Inverse of expectation — ≥ 5 mm scores 0.757, 2–3 mm scores 0.727 |

Table 2. Four factors tested and eliminated. The slice-thickness result is the reverse of the usual assumption, and argues against pursuing finer through-plane resolution.

One factor does predict performance, clearly and monotonically up to the median: gland size.

| Gland volume | n | Mean Dice | Median |
| --- | --- | --- | --- |
| < 2 630 mm³ | 49 | 0.676 | 0.721 |
| 2 630 – 3 295 mm³ | 50 | 0.755 | 0.772 |
| 3 295 – 4 150 mm³ | 50 | 0.776 | 0.799 |
| > 4 150 mm³ | 51 | 0.773 | 0.801 |

Table 3. The top three quartiles are flat; the smallest sits 0.10 below and contains every severe failure.

Lifting the smallest quartile to the level of the second would put the overall mean at 0.765. The gland occupies roughly 0.3% of a 384 × 384 field, so an in-plane region-of-interest cascade — localise, crop to 128–192 px, segment at full resolution — targets precisely the regime where performance falls away. This is the single change with direct evidence behind it.

# 4.  Post-processing: a result worth reporting in its own right

Connected-component post-processing contributes +0.002 Dice to this pipeline. In the 2025 pipeline the same operation contributes between +0.04 and +0.26. That difference is large enough to demand an explanation, and the explanation is measurable.

All six architectures in the 2025 paper operate per slice in 2D. Across its twenty-four model-by-gland-by-dataset entries, the post-processing gain correlates with raw Dice at r = −0.898: the weaker the raw model, the more pruning recovers. Extrapolating that relationship to a raw score of 0.746 predicts a gain of +0.130. This pipeline measures +0.002 — on the same dataset, the same organ and the same metric — and predicts only 2.2 to 2.7 connected components per gland, of which roughly one is removable.

Interpretation. The benefit widely attributed to connected-component pruning in these pipelines is largely the repair of inter-slice incoherence produced by per-slice 2D inference. A 2.5D formulation, which sees neighbouring slices, does not produce that incoherence and therefore has nothing for pruning to remove. A reported improvement is better described as the correction of an architectural defect.

A practical consequence follows. Selecting only the single largest connected component per gland can delete a correct prediction in favour of a larger incorrect one, producing Dice scores of exactly zero on cases the model had partly found. Retaining the two largest components removes that behaviour at a cost below the measurement resolution, and is the setting used here.

# 5.  Stage A: does slice gating work?

Yes, and this is the clearest positive result of the work. The best gate removes 63% of slices while retaining 99% of gland-bearing ones — a direct reduction in the segmentation workload with almost no loss of the tissue Stage B needs. Because a missed positive slice removes adrenal tissue permanently while a false positive costs only one wasted segmentation, the operating threshold is chosen post hoc as the highest threshold that still retains 99% of gland-bearing slices, and models are selected on PR-AUC, which is threshold-free.

Two controls make the architectural comparison interpretable. All arms share one sampler, augmentation scheme, schedule, seed, loss and metric, differing by a single flag. And parameter counts are matched: the recurrent baseline is configured to 281,826 parameters against the spiking gate's 285,666, within 2%. Both models also received the same tuning budget of nine configurations.

# 6.  Spiking versus recurrent: what the evidence shows

Two distinct questions were tested: whether the spiking gate outperforms the recurrent baseline, and whether the craniocaudal sequence contributes anything at all.

## The architectural comparison

|  | as specified | best of nine |  | gain from tuning | slices removed |
| --- | --- | --- | --- | --- | --- |
| Spiking gate | 0.7607 | 0.8407 |  | +0.0800 | 54.3% |
| Recurrent baseline | 0.8821 | 0.8827 |  | +0.0006 | 62.8% |

Table 4. PR-AUC, with retention held at 99% throughout.

The baseline leads by 0.042 PR-AUC and 8.5 percentage points of slice reduction after equal tuning effort. It also reached its ceiling immediately — nine configurations improved it by 0.0006 — while the spiking gate required the full budget to come within striking distance and still fell short. One mechanism is identified: replacing BatchNorm with GroupNorm was worth +0.051 to the spiking gate and +0.0001 to the baseline, on identical data and code paths, indicating that the train/eval mismatch created by normalising once per time step damages LIF dynamics in a way it does not damage ReLU.

## The sequence hypothesis

Three independent tests, all negative:

- At k = 5, five consecutive slices scored 0.7607 against 0.7572 for the same five time steps filled with the centre slice repeated — a difference of 0.0035.

- At k = 9, 0.8042 against 0.8086, with the repeated-slice control marginally ahead. Notably the control gains +0.051 from the longer window despite receiving no additional anatomy, indicating that extra time steps help computationally while extra anatomical context does not.

- On the baseline, averaging outputs across time steps — which discards most ordering information — costs 0.004.

A control experiment establishes that this is a property of the data, not of the models. In a synthetic task where a structure translates through the slice stack and the two classes contain identical slices in reversed order, so that ordering is the only available signal, every configuration tested reaches 100% accuracy. The architectures can represent slice order without difficulty. It follows that the ordering of adjacent slices carries little information about gland presence beyond the set of slices itself.

# 7.  Status of the evidence

Two qualifications apply to Section 6 and should be stated when these results are discussed. Every figure is from a single training run; seed replication across five seeds per configuration is in progress and will attach error bars and significance tests to each comparison. And both “best of nine” figures were selected on validation data, so each is optimistic by an unknown amount — symmetrically, which preserves the comparison between them, but the selection procedure needs describing alongside the numbers.

# 8.  What worked and what did not

## Worked

- Slice gating. A 63% reduction in slices at 99% retention of gland-bearing slices is a substantial and directly useful result.

- The 2.5D formulation, on measured evidence: 2.2–2.7 predicted components per gland against the fragmentation implied by 2D pipelines' post-processing gains.

- Batch-pooled Dice loss. With left and right as separate channels, many slices leave one channel's target empty; pooling numerator and denominator across the batch is necessary for the model to train at all.

- Matching capacity and tuning budget across arms, and holding the protocol otherwise fixed. This is what makes the Section 6 conclusion defensible rather than arguable.

## Did not work

- The spiking gate as a replacement for a recurrent baseline: behind by 0.042 PR-AUC after equal effort.

- The craniocaudal sequence encoding, on all three tests, with the architecture's capability ruled out as the cause.

- Threshold calibration as an explanation for the segmentation failure tail: worth +0.010.

- Finer through-plane resolution, which the slice-thickness stratification argues against before it was attempted.

- Single-largest-component post-processing, which can delete correct predictions.